# HoloLens CPR Training App - Setup and Usage Guide

## Part 1: Installation Instructions

### Prerequisites

- Microsoft HoloLens 2 device

- Microsoft account for HoloLens sign-in

- Stable Wi-Fi connection

- App installation package

### Installation Steps

- Power on the HoloLens

- Press and hold the power button for 2 seconds

- Sign in with Microsoft Account

- Follow the on-screen prompts to sign in

- Connect to Wi-Fi

- Settings -> Network & Internet -> Select your  Wi-Fi network

- Turn on Developer mode (If this is not done already)

- Settings -> Update and Security -> for developers and turn on Developer Mode

- Install the CPR App

- Connect your Holo lens 2 to your PC via USB-C cable

- Add files to the Holo lens 2

  - Unzip application file and drag + drop them into the Holo Lens 2

  - Do this in the Holo lens 2 tab found on your file explorer

- Install application

  - Go back to the Holo lens 2 interface and open the File Explorer found in “All apps”

  - On the left hand side select the all files icon. The file explorer puts you in the recent files menu by default, make sure to switch the menu to all files

  - Find the application folder with the same name as the one you dragged + dropped

  - Open it and click the “brown box”. This will begin the installation

  - Once opened enter the passcode for the device and click install found on the bottom right

- Run Application

- Once installation process has begun the application may start right away

- You can also find the application in the All Apps menu

- You may delete any old applications by holding down on their respective icons and clicking delete/remove

## Part 2: Using the CPR Training App

### Starting the Application

- Launch the App

  - Find and select “CPR Training App” from the app list

- Initial Setup

  - Select your role: Nurse or Doctor

### Understanding the Hand Menu

#### For Nurse Role (5 buttons):

- Pen Mode: Toggle between object manipulation and gaze interaction

- Sessions: Show/hide the session list and refresh active sessions

- Save Position: Save the current UI/object layout as a preset

- Load Position: Restore the previously saved layout preset

- Reset Position: Reset the layout back to default

#### For Doctor Role (7 buttons):

All nurse buttons plus: - Minimize Medication: Hide the medication panel - Maximize Medication: Show the medication panel

### Working with Holograms

#### Placing Holograms

- Initial Placement

  - Holograms will appear in front of you when the app starts

  - Use air tap and hold to grab a hologram

  - Move your hand to reposition

  - Release to place

- Adjusting Hologram Position

  - Enable Pen Mode from the hand menu

  - Look at the hologram you want to adjust

  - Air tap to select

  - Use the manipulation handles to:

    - Move (grab center)

    - Rotate (grab corners)

    - Resize (pinch corners)

- Setting Anchor Points

  - Position all holograms where desired

  - Click Save Position in the hand menu

  - This saves your layout for future sessions

#### Managing Sessions

- Viewing Active Sessions

  - Click Sessions button in hand menu

  - A list of available CPR sessions will appear

  - Air tap on a session to join

- Session Interface

  - Current task displays at the top

  - Next task shown below

  - Timer shows elapsed time

  - Medication status updates in real-time

#### CPR Procedure Elements

- Compressions Counter

  - Displays compression count

  - Shows compression rate

  - Visual feedback for depth/rate quality

- Medication Panel (Doctor only)

  - Shows medication schedule

  - Highlights when medications are due

  - Tap to confirm administration

- Notifications

  - Appear as floating text

  - Important alerts highlighted in red

  - Tap to dismiss non-critical notifications